TECHNICAL AND OPERATIONAL GUIDANCE (TECHOP)

TECHOP_ODP_04_(D)_(FMEA GAP ANALYSIS)

DECEMBER 2013

CONTENTS

SECTION	PAGE

1	INTRODUCTION - TECHOP (TECHNICAL AND OPERATIONAL GUIDANCE)	3

1.1	PREAMBLE	3

1.2	TECHOP_ODP	3

1.3	TECHOP_GEN	3

1.4	MTS DP GUIDANCE REVISION METHODOLOGY	3

2	SCOPE AND IMPACT OF THIS TECHOP	5

2.1	SCOPE	5

2.2	IMPACT ON PUBLISHED GUIDANCE	5

3	CASE FOR ACTION	6

3.1	WIDE VARIATION IN QUALITY AND SCOPE OF DP FMEAS	6

3.2	RECOMMENDED ACTION	6

4	SUGGESTED IMPLEMENTATION METHODOLOGY	7

4.1	INTRODUCTION	7

4.2	GUIDANCE FOR USE IN THE PREPARATION OF A GAP ANALYSIS	7

4.3	CARRYING OUT A GAP ANALYSIS	7

4.4	PROCESS FOR MITIGATING OUTPUT OF DP SYSTEM FMEA GAP ANALYSIS	9

4.5	CONCLUSION	11

5	MISCELLANEOUS	12

APPENDIX A	FMEA GAP ANALYSIS tABLE	13

A.1	REVIEW OF PREAMBLE	14

A.2	REVIEW OF CONTENT	15

# INTRODUCTION - TECHOP (TECHNICAL AND OPERATIONAL GUIDANCE)

## PREAMBLE

The Guidance documents on DP (Design and Operations) were published by the MTS DP Technical Committee in 2011 and 2010, Subsequent engagement has occurred with:

Classification Societies (DNV, ABS).

United States Coast Guard (USCG).

Marine Safety Forum (MSF).

Feedback has also been received through the comments section provided in the MTS DP Technical Committee Web Site.

It became apparent that a mechanism is needed to be developed and implemented to address the following in a pragmatic manner.

Feedback provided by the various stakeholders.

Additional information and guidance that the MTS DP Technical Committee wished to provide means to facilitate revisions to the documents and communication of the same to the various stakeholders.

The use of Technical and Operations Guidance Notes (TECHOP) was deemed to be a suitable vehicle to address the above.  These TECHOP Notes will be in two categories:

TECHOP_ODP.

TECHOP_GEN.

## TECHOP_ODP

Technical guidance Notes provided to address Guidance contained within the Operations, Design or People (Future development planned by the MTS DP Technical Committee) documents will be contained within this category.

The TECHOP will be identified by the following:

TECHOP_ODP_SNO_CATEGORY (DESIGN (D) OPERATIONS (O) PEOPLE (P).

EG 1 TECHOP_ODP_01_(O)_(HIGH LEVEL PHILOSOPHY).

EG 2 TECHOP_ODP_02_(D)_(BLACKOUT RECOVERY).

## TECHOP_GEN

MTS DP TECHNICAL COMMITTEE intends to publish topical white papers.  These topical white papers will be identified by the following:

TECHOP_GEN_SNO_DESCRIPTION.

EG 1 TECHOP_GEN_01-WHITE PAPER ON DP INCIDENTS.

EG 2 TECHOP_GEN_02-WHITE PAPER ON SHORT CIRCUIT TESTING.

## MTS DP GUIDANCE REVISION METHODOLOGY

TECHOP as described above will be published as relevant and appropriate.  These TECHOPs will be written in a manner that will facilitate them to be used as standalone documents.

Subsequent revisions of the MTS Guidance documents will review the published TECHOPs and incorporate as appropriate.

Communications with stakeholders will be established as appropriate to ensure that they are notified of intended revisions.  Stakeholders will be provided with the opportunity to participate in the review process and invited to be part of the review team as appropriate.

# SCOPE AND IMPACT OF THIS TECHOP

## SCOPE

TECHOP_ODP_04_(D)_(FMEA GAP ANALYSIS). This TECHOP addresses the wide variation in the scope and quality of DP FMEAs for DP vessels of all types entering service or following conversion for dynamic positioning.

## IMPACT ON PUBLISHED GUIDANCE

This TECHOP impacts MTS DP Vessel Design Philosophy Guidelines Part II, Section 22.

# CASE FOR ACTION

## WIDE VARIATION IN QUALITY AND SCOPE OF DP FMEAS

All dynamically positioned vessels of DP Equipment Classes 2 & 3 are required to be single fault tolerant in respect of defined failure criteria appropriate to the equipment Class. Fault tolerance should be demonstrated to the satisfaction of the Classification society. These requirements are traditionally satisfied by carrying out a Failure Modes and Effects Analysis (FMEA) which is validated by DP FMEA proving trials.

There is wide variation in the quality and scope of FMEAs being approved as demonstrating such fault tolerance. Failure to detect single point failures and vulnerability to common cause failures in the design of DP systems leaves the vessel vulnerable to loss of position and heading when the undetected failure mode occurs while carrying out DP operations.

The potential consequences of such failures depend on the nature of the work but typically include:

Damage to the DP vessel and/or vessels operating in the vicinity.

Damage to infrastructure and subsea installations.

Damage to the environment.

Financial losses.

Risk to life.

Loss of reputation.

Even when a poor quality FMEA is not approved and is rejected by Class the fact that a significant amount of design and construction work has often been completed by the time the FMEA is rejected means that there may be a need for significant and costly rework to correct issues not identified by the FMEA. This can delay vessel delivery and threaten contracts if the vessel is not available to perform the work on time.

Classification societies recognise the importance of a good quality FMEA and the significant risks associated with an incompetent analysis and trials program. Guidance is now available in the form of a recommended practice for ‘FMEA of Redundant Systems’. Such guidance goes a long way to addressing the shortfalls but is not yet universally applied.

Note: In this context the term ‘DP system’ implies all equipment involved in maintaining position and heading including power generation propulsion and control systems. This is the official definition from IMO MSC 645 although the term ‘DP system’ is sometimes used informally to refer to the DP control system.

## RECOMMENDED ACTION

FMEA stakeholders should review and comment upon FMEAs submitted as part of the approval process. The most effective and objective way to confirm whether a DP FMEA is competent is to carry out a formal gap analysis between the document submitted and the industry guidance on FMEAs referenced in this TECHOP.

A significant deviation from the scope and methodology recommended in the referenced guidance indicates that the FMEA may not have been executed in a competent manner and the DP vessel may not have the required level of fault tolerance.

FMEA stakeholders should then ensure the identified deficiencies are addressed which may require comprehensive rework of the FMEA and proving trials.

# SUGGESTED IMPLEMENTATION METHODOLOGY

## INTRODUCTION

### The purpose of this TECHOP is to help ensure FMEAs for DP Vessels:

Are competently executed.

Analyse all DP related systems.

Consider the influence of the industrial mission on the DP redundancy concept.

Are effectively validated by the DP FMEA proving trials.

## GUIDANCE FOR USE IN THE PREPARATION OF A GAP ANALYSIS

There are several sources of information on the scope and details of failure modes and effects analysis:

MTS DP Vessel Design Philosophy Guidelines Part II.

DNV Recommended Practice for FMEA of Redundant Systems, RP-D102.

IMCA Guidance M166 – ‘Guidance on Failure Modes and Effects Analysis’, April 2002.

IMCA M206 A Guide to DP Electrical Power and Control Systems.

IMCA Information Note M04/04 - IMCA Study – ‘Methods of Establishing the Safety and Reliability of DP Systems’ – 2004, Appendices D & E in particular.

New sources of guidance are published periodically and further revision of the MTS DP Vessel Design Philosophy Guidelines will reference these sources as they become available. The sources of guidance listed above have been used to prepare the gap analysis table in Appendix A.

## CARRYING OUT A GAP ANALYSIS

The purpose of the gap analysis is to check the scope and methodology used in the DP FMEA against published industry guidance and not to repeat or correct the analysis. The gap analysis is typically carried out using only the FMEA report itself as source material.

A FMEA gap analysis can be performed using the table in Appendix A. The large number of issues to be checked is indicative of the complexity of modern DP vessels and the number of systems, subsystems and equipment items that influence the redundancy concept.  Checklist items related to the correct application of FMEA methodology are repeated in each subsystem which creates repetition but to restrict these issues to a single table entry is to risk overlooking essential elements of the analysis.

A simple colour coding scheme can be used to identify whether the FMEA issue in the table have been satisfactorily addressed, partially addressed, omitted completely or contains significant errors. Grey can be used to indicate issues that do not apply to the design of the subject DP vessel.

Green – Analysis satisfactory.

Yellow – Analysis incomplete.

Red - Analysis unsatisfactory (analysis omitted or contains significant errors).

Grey – Not applicable.

An example of a significant error would be presenting incorrect failure effects or omitting to analyse a failure mode with effects exceeding the worst case failure design intent. This is not intended to imply a re-analysis of the original design only that there are demonstrable errors in the FMEA.

The gap analysis table includes two types of issues to be checked:

Issues associated with application of DP FMEA methodology.

Lists of equipment and systems which would typically be discussed in a DP FMEA.

A DP FMEA should have sufficient information to allow a complete understanding of the redundancy concept. However, in some cases, when considering items that should be discussed in a DP FMEA it may not be obvious from the FMEA report whether the equipment is not fitted to this particular vessel or it is fitted but the analysis has been omitted. The following rules may be applied to determine the category to assign in the gap analysis table:

If the omission could conceal failure effects of severity greater than the worst case failure design intent it would be categorised as RED.

If the omission would have no significant impact on the severity of failure effects then the issue may be categorised as GREY.

For example, the absence of discussion of a remote valve control systems may mean that a single point failure associated with all cooling valves failing to the closed position has been overlooked particularly if there is no explicit statement indicating that all valves are manually controlled. However, the absence of the discussion of fuel oil coolers in a system with independent fuel and cooling water systems for each redundant machinery group is unlikely to be significant.

Once the grey entries have been discounted, the ratio of yellows and reds to the overall number of issues can be used to provide an indication of the level of deficiency in percentage terms.

A summary report discussing the problems and major deficiencies can also be provided along with suggestions on how to remedy the analysis and complete it to the required level.

Four columns in the table indicate the DP system arrangements to which each issue applies: There are typically entries in several columns for each issue.

- Closed busties: A tick in this column indicates that issues should be discussed in FMEAs for all Classes of vessel (2 & 3) that operate their power plant as a single common power system.

- Open busties: A tick in this column Indicates that the issue should be considered for all Classes of vessels that operate their power plant as two or more isolated systems.

- DP Class 3 An entry in this column indicates the issue should be considered for DP Class 3 designs. If this is the only entry against a particular issue then it is typically an issue associated with the effects of fire and flooding.

- Fail Safe: An entry in this column indicates the issue is related to systems which can fail in such a way as cause a drive off, typically thruster controls systems or DP control system, references and sensors.

Note: The most useful time to carry out a FMEA gap analysis is on the draft revision of the FMEA when it is submitted to the vessel owner for review prior to submission to Class for approval. It may be difficult to influence further development after Class has approved it with comments. A similar process can be applied to the DP FMEA proving trials and annual DP trials programs.

## PROCESS FOR MITIGATING OUTPUT OF DP SYSTEM FMEA GAP ANALYSIS

Figure 4-1 provides a flowchart which describes the process for mitigating the output of the DP system FMEA gap analysis. There are essentially six steps to be completed if the gap analysis indicates that action is required:

- Carry out the gap analysis.

Confirm any yellow and red findings by reference to the original vessel design documentation.

Perform additional Failure Modes and Effects Analysis to address the gaps.

If unacceptable failure effects are identified then there are two courses of action depending on whether the vessel is a newbuild or already in service.

If he vessel is already in service at the time of the gap analysis:

    - Develop effective means to mitigate the risks based on the key elements of Design, Operations and People (or combinations of these)

    - Document the mitigations and evaluate the potential for updating the vessels DP system FMEA as part of the FMEA management process.

    - Manage the mitigating measures.

- If the vessel is a new-build awaiting proving trials then the DP FMEA and proving trials program should be updated to remedy the deficiencies before the vessel enters service. This process may identify the need for system modifications.

Figure 4-1	Process for Mitigating Output of Gap Analysis

## CONCLUSION

There is very significant variation in the quality and scope of DP vessel FMEAs currently being approved.  The FMEA gap analysis is a useful quality indicator and should be carried out whenever there is concern regarding the scope, competence and veracity of a FMEA. There is a significant amount of useful guidance on FMEAs and DP system design against which any analysis can be judged.

# MISCELLANEOUS

| Stakeholders | Impacted | Remarks |
| --- | --- | --- |
| MTS DP Committee |  | To track and incorporate in next rev of MTS DP Operations Guidance Document Part 2 Appendix 1. Communicate to DNV, USCG, Upload in MTS website. |
| USCG |  | MTS to communicate- FR notice impacted when Rev is available. |
| DNV | X | MTS to Communicate- DNV RP E 307 impacted. |
| Equipment vendor community |  | MTS to engage with protection suppliers |
| Consultant community |  | MTS members to cascade/ promulgate. |
| Training institutions | X | MTS members to cascade/ promulgate. |
| Vessel Owners/Operators |  | Establish effective means to disseminate information to Vessel Management and Vessel Operational Teams. |
| Vessel Management/Operational teams |  | Establish effective means to disseminate information to Vessel Operational Teams. |

FMEA GAP ANALYSIS tABLE

REVIEW OF PREAMBLE

| DP FMEA GAP ANALYSIS - FMEA DOCUMENT NUMBER AXXXXX-X |  |  |  |  |
| --- | --- | --- | --- | --- |
| SYSTEM | ID NO. | CROSS REFERENCE TO FMEA | YES / PARTIAL / NO / NOT APPLICABLE GREEN / YELLOW / RED / GREY | CONCERNS |
| Applicable rules | 1 |  |  |  |
| Vessel class notation | 2 |  |  |  |
| FMEA standards | 3 |  |  |  |
| Main machinery outline | 4 |  |  |  |
| Critical activity mode identified | 5 |  |  |  |
| Redundancy concept overview | 6 |  |  |  |
| Redundant levels of power | 7 |  |  |  |
| Previous FMEA analysis | 8 |  |  |  |
| Vessel modifications | 9 |  |  |  |
| Worst case failure design intent | 10 |  |  |  |
| Summary/conclusion | 11 |  |  |  |
| Action items | 12 |  |  |  |
| Reference to all software revisions in use at time of proving trials | 13 |  |  |  |
| Reference to DP FMEA proving trials document | 14 |  |  |  |
| System drawings | 15 |  |  |  |

REVIEW OF CONTENT

| DP FMEA GAP ANALYSIS - FMEA DOCUMENT NUMBER AXXXXX-X |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DESCRIPTION |  |  |  | APPLICATION |  |  |  | GAP ANALYSIS |  |  |
| SYSTEM | SUB SYSTEM | ITEMS FOR ANALYSIS | ID NO. | CLOSED BUSTIE OPERATION | OPEN BUSTIE OPERATION | DP CLASS 3 | FAIL SAFE | CROSS REFERENCE TO FMEA | YES / PARTIAL / NO / NOT APPLICABLE GREEN / YELLOW / RED / GREY | CONCERN |
| Marine Auxiliary Systems | Fuel | Document reference with revision numbers. | 1 |  |  |  |  |  |  |  |
|  |  | Location for DP Class 3. | 2 |  |  |  |  |  |  |  |
|  |  | Description of fuel system and redundancy concept. | 3 |  |  |  |  |  |  |  |
|  |  | System configuration for DP. | 4 |  |  |  |  |  |  |  |
|  |  | Analysis identifies redundant groups in fuel system. | 5 |  |  |  |  |  |  |  |
|  |  | Analysis identifies common points between redundant groups. | 6 |  |  |  |  |  |  |  |
|  |  | Analysis identifies all failure effects that can propagate from one redundant group to the other. | 7 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any protective functions upon which redundancy depends to prevent failure effects propagating. | 8 |  |  |  |  |  |  |  |
| Marine Auxiliary Systems | Fuel | Analysis identifies any potential hidden failure that could defeat redundancy, including any alarms or standby redundancy. | 9 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any internal common cause failures and the barriers employed to mitigate them. | 10 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any external common cause failure and the barriers used to mitigate them. | 11 |  |  |  |  |  |  |  |
|  |  | The analysis identifies any configuration errors that could defeat redundancy and any barrier to prevent this. | 12 |  |  |  |  |  |  |  |
|  |  | There is a system drawing that identifies all DP related components, the redundant groups to which they belong and, their interfaces to other systems. | 13 |  |  |  |  |  |  |  |
| Marine Auxiliary Systems | Fuel | The worst case failure for those systems is clearly stated in relation to its effects on generators, thrusters, position and heading. | 14 |  |  |  |  |  |  |  |
|  |  | There is a link to the proving trials program that confirms the findings of the FMEA. | 15 |  |  |  |  |  |  |  |
|  |  | Protection against damage for passive components where exempt from failure analysis as part of redundancy concept for DP Class 2. | 16 |  |  |  |  |  |  |  |
|  |  | A60 separation of redundant groups. | 17 |  |  |  |  |  |  |  |
|  |  | Watertight separation of redundant groups. | 18 |  |  |  |  |  |  |  |
|  |  | A60 separation of pipework for redundant groups for DP Class 3. | 19 |  |  |  |  |  |  |  |
|  |  | Watertight separation of pipework. | 20 |  |  |  |  |  |  |  |
|  |  | Storage tanks free of ballast lines etc. | 21 |  |  |  |  |  |  |  |
| Marine Auxiliary Systems | Fuel | Storage and transfer facilities allow fuel to be quarantined. | 22 |  |  |  |  |  |  |  |
|  |  | Water detection. | 23 |  |  |  |  |  |  |  |
|  |  | Day/service/settling tanks. | 24 |  |  |  |  |  |  |  |
|  |  | Tank level indications. | 25 |  |  |  |  |  |  |  |
|  |  | Quick closing valves and their panels/levers. | 26 |  |  |  |  |  |  |  |
|  |  | Fuel modules including DO to HO changeover. | 27 |  |  |  |  |  |  |  |
|  |  | HFO fuel heating – Steam / technical oil. | 28 |  |  |  |  |  |  |  |
|  |  | Automated valves. | 29 |  |  |  |  |  |  |  |
|  |  | Pumps, mechanical and electrical. | 30 |  |  |  |  |  |  |  |
|  |  | Purifiers. | 31 |  |  |  |  |  |  |  |
|  |  | Flow meters, supply and return. | 32 |  |  |  |  |  |  |  |
|  |  | Filters, suction and discharge | 33 |  |  |  |  |  |  |  |
| Marine Auxiliary Systems | Fuel | Low level alarms. | 34 |  |  |  |  |  |  |  |
|  |  | Pressure alarms. | 35 |  |  |  |  |  |  |  |
|  |  | Supply lines. | 36 |  |  |  |  |  |  |  |
|  |  | Return lines with valves. | 37 |  |  |  |  |  |  |  |
|  |  | Fuel coolers – leaks from cooling water to fuel. | 38 |  |  |  |  |  |  |  |
|  |  | Electrical supplies to pumps and automation. | 39 |  |  |  |  |  |  |  |
|  |  | Valve actuation. | 40 |  |  |  |  |  |  |  |
|  |  | Effects of most common leaks. | 41 |  |  |  |  |  |  |  |
|  |  | Leak protection. | 42 |  |  |  |  |  |  |  |
|  |  | Leak alarms. | 43 |  |  |  |  |  |  |  |
|  |  | Flexible lines. | 44 |  |  |  |  |  |  |  |
|  |  | Hot surface protection. | 45 |  |  |  |  |  |  |  |
| Marine Auxiliary Systems | Remote Valve Control | Document reference with revision numbers. | 46 |  |  |  |  |  |  |  |
|  |  | Location for DP Class 3. | 47 |  |  |  |  |  |  |  |
|  |  | Description of RCV system and redundancy concept. | 48 |  |  |  |  |  |  |  |
|  |  | Analysis identifies redundant groups in RCV system. | 49 |  |  |  |  |  |  |  |
|  |  | Analysis identifies common points between redundant groups. | 50 |  |  |  |  |  |  |  |
|  |  | Analysis identifies all failure effects that can propagate from one redundant group to the other. | 51 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any protective functions upon which redundancy depends to prevent failure effects propagating. | 52 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Remote Valve Control | Analysis identifies any potential hidden failure that could defeat redundancy, including any alarms or standby redundancy. | 53 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any internal common cause failures and the barriers employed to mitigate them. | 54 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any external common cause failure and the barriers used to mitigate them. | 55 |  |  |  |  |  |  |  |
|  |  | The analysis identifies any configuration errors that could defeat redundancy and any barrier to prevent this. | 56 |  |  |  |  |  |  |  |
|  |  | There is a system drawing that identifies all DP related components, the redundant groups to which they belong and, their interfaces to other systems. | 57 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Remote Valve Control | The worst case failure for those systems is clearly stated in relation to its effects on generators, thrusters, position and heading. | 58 |  |  |  |  |  |  |  |
|  |  | There is a link to the proving trials program that confirms the findings of the FMEA. | 59 |  |  |  |  |  |  |  |
|  |  | Protection against damage for passive components where exempt from failure analysis as part of redundancy concept for DP Class 2. | 60 |  |  |  |  |  |  |  |
|  |  | A60 separation of redundant groups | 61 |  |  |  |  |  |  |  |
|  |  | Watertight separation of redundant groups | 62 |  |  |  |  |  |  |  |
|  |  | A60 separation of pipework for redundant groups for DP Class 3. | 63 |  |  |  |  |  |  |  |
|  |  | Generator CW valves 'fail as set'. | 64 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Remote Valve Control | Valves for DP essential services fail safe. | 65 |  |  |  |  |  |  |  |
|  | Engine & Generator Lubricating Oil (see also Thrusters) | Document reference with revision numbers. | 66 |  |  |  |  |  |  |  |
|  |  | Location for DP Class 3. | 67 |  |  |  |  |  |  |  |
|  |  | Description of lubricating system and redundancy concept. | 68 |  |  |  |  |  |  |  |
|  |  | System configuration for DP. | 69 |  |  |  |  |  |  |  |
|  |  | Analysis identifies redundant groups in lubrication system. | 70 |  |  |  |  |  |  |  |
|  |  | Analysis identifies common points between redundant groups. | 71 |  |  |  |  |  |  |  |
|  |  | Analysis identifies all failure effects that can propagate from one redundant group to the other. | 72 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Engine & Generator Lubricating Oil (see also Thrusters) | Analysis identifies any protective functions upon which redundancy depends to prevent failure effects propagating. | 73 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any potential hidden failure that could defeat redundancy, including any alarms or standby redundancy | 74 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any internal common cause failures and the barriers employed to mitigate them. | 75 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any external common cause failure and the barriers used to mitigate them. | 76 |  |  |  |  |  |  |  |
|  |  | The analysis identifies any configuration errors that could defeat redundancy and any barrier to prevent this. | 77 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Engine & Generator Lubricating Oil (see also Thrusters) | There is a system drawings that identifies all DP related components, the redundant groups to which they belong and their interfaces to other systems. | 78 |  |  |  |  |  |  |  |
|  |  | The worst case failure for those systems is clearly stated in relation to its effects on generators, thrusters, position and heading. | 79 |  |  |  |  |  |  |  |
|  |  | There is a link to the proving trials program that confirms the findings of the FMEA. | 80 |  |  |  |  |  |  |  |
|  |  | Protection against damage for passive components where exempt from failure analysis as part of redundancy concept for DP Class 2. | 81 |  |  |  |  |  |  |  |
|  |  | A60 separation of redundant groups. | 82 |  |  |  |  |  |  |  |
|  |  | Watertight separation of redundant groups. | 83 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Engine & Generator Lubricating Oil (see also Thrusters) | A60 separation of pipework for redundant groups for DP Class 3. | 84 |  |  |  |  |  |  |  |
|  |  | Storage. | 85 |  |  |  |  |  |  |  |
|  |  | Transfer. | 86 |  |  |  |  |  |  |  |
|  |  | Filling. | 87 |  |  |  |  |  |  |  |
|  |  | Sump arrangement. | 88 |  |  |  |  |  |  |  |
|  |  | Pumps, mechanical and electrical. | 89 |  |  |  |  |  |  |  |
|  |  | Level alarms and watch-keeping procedures. | 90 |  |  |  |  |  |  |  |
|  |  | Low pressure alarms. | 91 |  |  |  |  |  |  |  |
|  |  | Alarm and shutdown modes. | 92 |  |  |  |  |  |  |  |
|  |  | Alternator bearings. | 93 |  |  |  |  |  |  |  |
|  |  | Filters. | 94 |  |  |  |  |  |  |  |
|  |  | Coolers. | 95 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Engine & Generator Lubricating Oil (see also Thrusters) | Automated valves. | 96 |  |  |  |  |  |  |  |
|  |  | Purification and cross connection of generators. | 97 |  |  |  |  |  |  |  |
|  |  | Crank case ventilation – commonality in pipework & spark arresters. | 98 |  |  |  |  |  |  |  |
|  |  | Oil mist detectors active shutdown and common air supply. | 99 |  |  |  |  |  |  |  |
|  |  | Priming pumps and readiness for standby starts. | 100 |  |  |  |  |  |  |  |
|  |  | Electrical supplies to pumps and automation. | 101 |  |  |  |  |  |  |  |
|  |  | Pre-lubrication for BOR – independence of emergency generator. Override for pre-lubrication. | 102 |  |  |  |  |  |  |  |
|  | Seawater Cooling | Document reference with revision numbers. | 103 |  |  |  |  |  |  |  |
|  |  | Location for DP Class 3. | 104 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Seawater Cooling | Description of seawater cooling systems and redundancy concept. | 105 |  |  |  |  |  |  |  |
|  |  | System configuration for DP. | 106 |  |  |  |  |  |  |  |
|  |  | Analysis identifies redundant groups in seawater system. | 107 |  |  |  |  |  |  |  |
|  |  | Analysis identifies all failure effects that can propagate from one redundant group to the other. | 108 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any protective functions upon which redundancy depends to prevent failure effects propagating. | 109 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any potential hidden failure that could defeat redundancy, including any alarms or standby redundancy. | 110 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Seawater Cooling | Analysis identifies any internal common cause failures and the barriers employed to mitigate them. | 111 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any external common cause failure and the barriers used to mitigate them. | 112 |  |  |  |  |  |  |  |
|  |  | The analysis identifies any configuration errors that could defeat redundancy and any barrier to prevent this. | 113 |  |  |  |  |  |  |  |
|  |  | There is a system drawing that identifies all DP related components, the redundant groups to which they belong and, and their interfaces to other systems. | 114 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Seawater Cooling | The worst case failure for those systems is clearly stated in relation to its effects on generators, thrusters, position and heading. | 115 |  |  |  |  |  |  |  |
|  |  | There is a link to the proving trials program that confirms the findings of the FMEA. | 116 |  |  |  |  |  |  |  |
|  |  | Protection against damage for passive components where exempt from failure analysis as part of redundancy concept for DP Class 2. | 117 |  |  |  |  |  |  |  |
|  |  | A60 separation of redundant groups. | 118 |  |  |  |  |  |  |  |
|  |  | Watertight separation of redundant groups. | 119 |  |  |  |  |  |  |  |
|  |  | A60 separation of pipework for redundant groups for DP Class 3. | 120 |  |  |  |  |  |  |  |
|  |  | Bulkhead valves either side for DP Class 3. | 121 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Seawater Cooling | Sea chests. | 122 |  |  |  |  |  |  |  |
|  |  | Seawater pressure alarm. | 123 |  |  |  |  |  |  |  |
|  |  | Strainer differential pressure alarm. | 124 |  |  |  |  |  |  |  |
|  |  | Seawater flow alarm. | 125 |  |  |  |  |  |  |  |
|  |  | Sea strainers. | 126 |  |  |  |  |  |  |  |
|  |  | Anti-fouling - biocide treatment. | 127 |  |  |  |  |  |  |  |
|  |  | Remote/Automatic valves, actuation and control. | 128 |  |  |  |  |  |  |  |
|  |  | Emergency shell isolation, actuation and control. | 129 |  |  |  |  |  |  |  |
|  |  | Overboard and valves (common?) | 130 |  |  |  |  |  |  |  |
|  |  | Pumps, mechanical and electrical. | 131 |  |  |  |  |  |  |  |
|  |  | Electrical supplies to pumps and automation. | 132 |  |  |  |  |  |  |  |
|  |  | Recirculation to sea chests. | 133 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Seawater Cooling | Winterisation. | 134 |  |  |  |  |  |  |  |
|  |  | Blow downs. | 135 |  |  |  |  |  |  |  |
|  |  | Coolers. | 136 |  |  |  |  |  |  |  |
|  |  | Emergency cross-connection from fire main etc. | 137 |  |  |  |  |  |  |  |
|  |  | Leaks – bilge alarms etc. | 138 |  |  |  |  |  |  |  |
|  | Freshwater Cooling | Document reference with revision numbers. | 139 |  |  |  |  |  |  |  |
|  |  | Location for DP Class 3. | 140 |  |  |  |  |  |  |  |
|  |  | Description of fresh water cooling systems and redundancy concept. | 141 |  |  |  |  |  |  |  |
|  |  | System configuration for DP. | 142 |  |  |  |  |  |  |  |
|  |  | Analysis identifies redundant groups in freshwater system. | 143 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Freshwater Cooling | Analysis identifies common points between redundant groups. | 144 |  |  |  |  |  |  |  |
|  |  | Analysis identifies all failure effects that can propagate from one redundant group to the other. | 145 |  |  |  |  |  |  |  |
|  |  | Analysis identified any protective functions upon which redundancy depends to prevent failure effects propagating. | 146 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any potential hidden failure that could defeat redundancy, including any alarms or standby redundancy. | 147 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any internal common cause failures and the barriers employed to mitigate them. | 148 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Freshwater Cooling | Analysis identifies any external common cause failure and the barriers used to mitigate them. | 149 |  |  |  |  |  |  |  |
|  |  | The analysis identifies any configuration errors that could defeat redundancy and any barrier to prevent this. | 150 |  |  |  |  |  |  |  |
|  |  | There is a system drawing that identifies all DP related components, the redundant groups to which they belong and, and their interfaces to other systems. | 151 |  |  |  |  |  |  |  |
|  |  | The worst case failure for those systems is clearly stated in relation to its effects on generators, thrusters, position and heading. | 152 |  |  |  |  |  |  |  |
|  |  | There is a link to the proving trials program that confirms the findings of the FMEA. | 153 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Freshwater Cooling | Protection against damage for passive components where exempt from failure analysis as part of redundancy concept for DP Class 2. | 154 |  |  |  |  |  |  |  |
|  |  | A60 separation of redundant groups. | 155 |  |  |  |  |  |  |  |
|  |  | Watertight separation of redundant groups | 156 |  |  |  |  |  |  |  |
|  |  | A60 separation of pipework for redundant groups for DP Class 3. | 157 |  |  |  |  |  |  |  |
|  |  | Bulkhead valves either side for DP Class 3. | 158 |  |  |  |  |  |  |  |
|  |  | Fuel valve cooling. | 159 |  |  |  |  |  |  |  |
|  |  | LTFW – TLVs power and signal source. | 160 |  |  |  |  |  |  |  |
|  |  | HTFW – TLVs power and signal source. | 161 |  |  |  |  |  |  |  |
|  |  | Pumps, mechanical and electrical duty and standby. | 162 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Freshwater Cooling | Header tanks (commonality for Class 3). | 163 |  |  |  |  |  |  |  |
|  |  | Electrical supplies for pumps. | 164 |  |  |  |  |  |  |  |
|  |  | Thermostatic valves. | 165 |  |  |  |  |  |  |  |
|  |  | Coolers – burst, blocked, cross leak. | 166 |  |  |  |  |  |  |  |
|  |  | Water makers – Vacuum, heaters, CW supply. | 167 |  |  |  |  |  |  |  |
|  |  | Remote/Automatic valves. | 168 |  |  |  |  |  |  |  |
|  |  | System cross-connection. | 169 |  |  |  |  |  |  |  |
|  |  | Tank level alarms. | 170 |  |  |  |  |  |  |  |
|  |  | High temperature alarms and shutdowns. | 171 |  |  |  |  |  |  |  |
|  |  | Leaks. | 172 |  |  |  |  |  |  |  |
|  |  | Flexible pipes. | 173 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Charge Air | Document reference with revision numbers. | 174 |  |  |  |  |  |  |  |
|  |  | Location for DP Class 3. | 175 |  |  |  |  |  |  |  |
|  |  | Description of charge air systems and redundancy concept. | 176 |  |  |  |  |  |  |  |
|  |  | System configuration for DP. | 177 |  |  |  |  |  |  |  |
|  |  | Analysis identifies redundant groups in charge-air system. | 178 |  |  |  |  |  |  |  |
|  |  | Analysis identifies common points between redundant groups. | 179 |  |  |  |  |  |  |  |
|  |  | Analysis identifies all failure effects that can propagate from one redundant group to the other. | 180 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any protective functions upon which redundancy depends to prevent failure effects propagating. | 181 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Charge Air | Analysis identifies any potential hidden failure that could defeat redundancy, including any alarms or standby redundancy. | 182 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any internal common cause failures and the barriers employed to mitigate them. | 183 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any external common cause failure and the barriers used to mitigate them. | 184 |  |  |  |  |  |  |  |
|  |  | The analysis identifies any configuration errors that could defeat redundancy and any barrier to prevent this. | 185 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Charge Air | There is a system drawing that identifies all DP related components, the redundant groups to which they belong and, and their interfaces to other systems. | 186 |  |  |  |  |  |  |  |
|  |  | The worst case failure for those systems is clearly stated in relation to its effects on generators, thrusters, position and heading. | 187 |  |  |  |  |  |  |  |
|  |  | There is a link to the proving trials program that confirms the findings of the FMEA. | 188 |  |  |  |  |  |  |  |
|  |  | Protection against damage for passive components where exempt from failure analysis as part of redundancy concept for DP Class 2. | 189 |  |  |  |  |  |  |  |
|  |  | A60 separation of redundant groups. | 190 |  |  |  |  |  |  |  |
|  |  | Watertight separation of redundant groups. | 191 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Charge Air | A60 separation of pipework for redundant groups for DP Class 3. | 192 |  |  |  |  |  |  |  |
|  |  | Bulkhead valves either side for DP Class 3. | 193 |  |  |  |  |  |  |  |
|  |  | Turbochargers. | 194 |  |  |  |  |  |  |  |
|  |  | Rig savers – failure modes source of power and control. | 195 |  |  |  |  |  |  |  |
|  |  | Dump valves. | 196 |  |  |  |  |  |  |  |
|  |  | Jet assist – failure effects, air supply. | 197 |  |  |  |  |  |  |  |
|  |  | Coolers, burst blocked, cross leak. | 198 |  |  |  |  |  |  |  |
|  |  | Air temperature control. | 199 |  |  |  |  |  |  |  |
|  |  | Control valves, including actuation and control. | 200 |  |  |  |  |  |  |  |
|  | Compressed Air | Document reference with revision numbers. | 201 |  |  |  |  |  |  |  |
|  |  | Location for DP Class 3. | 202 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Compressed Air | Description of compressed air systems and redundancy concept. | 203 |  |  |  |  |  |  |  |
|  |  | Configuration for DP. | 204 |  |  |  |  |  |  |  |
|  |  | Analysis identifies redundant groups in compressed air system. | 205 |  |  |  |  |  |  |  |
|  |  | Analysis identifies common points between redundant groups. | 206 |  |  |  |  |  |  |  |
|  |  | Analysis identifies all failure effects that can propagate from one redundant group to the other. | 207 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any protective functions upon which redundancy depends to prevent failure effects propagating. | 208 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Compressed Air | Analysis identifies any potential hidden failure that could defeat redundancy, including any alarms or standby redundancy. | 209 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any internal common cause failures and the barriers employed to mitigate them. | 210 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any external common cause failure and the barriers used to mitigate them | 211 |  |  |  |  |  |  |  |
|  |  | The analysis identifies any configuration errors that could defeat redundancy and any barrier to prevent this | 212 |  |  |  |  |  |  |  |
|  |  | There is a system drawing that identifies all DP related components, the redundant groups to which they belong and, and their interfaces to other systems. | 213 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Compressed Air | The worst case failure for those systems is clearly stated in relation to its effects on generators, thrusters, position and heading. | 214 |  |  |  |  |  |  |  |
|  |  | There is a link to the proving trials program that confirms the findings of the FMEA. | 215 |  |  |  |  |  |  |  |
|  |  | Protection against damage for passive components where exempt from failure analysis as part of redundancy concept for DP Class 2. | 216 |  |  |  |  |  |  |  |
|  |  | A60 separation of redundant groups. | 217 |  |  |  |  |  |  |  |
|  |  | Watertight separation of redundant groups. | 218 |  |  |  |  |  |  |  |
|  |  | A60 separation of pipework for redundant groups for DP Class 3. | 219 |  |  |  |  |  |  |  |
|  |  | Bulkhead valves either side for DP Class 3. | 220 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Compressed Air | Start air. | 221 |  |  |  |  |  |  |  |
|  |  | Compressors. | 222 |  |  |  |  |  |  |  |
|  |  | Receivers. | 223 |  |  |  |  |  |  |  |
|  |  | Operating modes. | 224 |  |  |  |  |  |  |  |
|  |  | Cross-connections. | 225 |  |  |  |  |  |  |  |
|  |  | Reducing valves failure to zero / overpressure. | 226 |  |  |  |  |  |  |  |
|  |  | Electrical supplies. | 227 |  |  |  |  |  |  |  |
|  |  | Compressor automation – lead, lag. | 228 |  |  |  |  |  |  |  |
|  |  | Pressure alarms. | 229 |  |  |  |  |  |  |  |
|  |  | Emergency start air connection. | 230 |  |  |  |  |  |  |  |
|  |  | Leaks. | 231 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Emergency Generator | Document reference. | 232 |  |  |  |  |  |  |  |
|  |  | Analysis confirms no E-gen involvement in DP redundancy concept. | 233 |  |  |  |  |  |  |  |
|  |  | Interface with DP related equipment. | 234 |  |  |  |  |  |  |  |
|  |  | Mode indication -emergency/harbour. | 235 |  |  |  |  |  |  |  |
|  |  | Role in BOR. | 236 |  |  |  |  |  |  |  |
|  | Engine Management and Safety System | Document reference. | 237 |  |  |  |  |  |  |  |
|  |  | Description of engine control and safety systems. | 238 |  |  |  |  |  |  |  |
|  |  | Analysis confirms independence of control and safety functions. | 239 |  |  |  |  |  |  |  |
|  |  | Cross connections between safety systems. | 240 |  |  |  |  |  |  |  |
|  |  | Power supply segregation. | 241 |  |  |  |  |  |  |  |
|  |  | VMS interface. | 242 |  |  |  |  |  |  |  |
| Marine Auxiliary System | Engine Management and Safety System | Shutdowns – susceptible to common mode failures? (Oil mist, crankcase pressure). | 243 |  |  |  |  |  |  |  |
|  |  | Switchboard interface- MODBUS etc. | 244 |  |  |  |  |  |  |  |
|  |  | Emergency load up ramps. | 245 |  |  |  |  |  |  |  |
|  |  | Emergency shutdown – commonality. | 246 |  |  |  |  |  |  |  |
|  |  | Slow turning – effect on standby start times. | 247 |  |  |  |  |  |  |  |
|  |  | Start interlocks. | 248 |  |  |  |  |  |  |  |
|  |  | Starting and connection time – PMS supervision of ‘Hot Standby / Cold Standby) | 249 |  |  |  |  |  |  |  |
|  |  | Engine failure modes e.g. trip on RP, Slow down and stop, over power/ speed, hunting, catastrophic mechanical failure. | 250 |  |  |  |  |  |  |  |
| Power Generation | Generators | Document reference. | 251 |  |  |  |  |  |  |  |
|  |  | Location for DP 3. | 252 |  |  |  |  |  |  |  |
|  |  | Description of generator, type, speed, Hz, current, pf, voltage, kW, kVA. Excitation system, PMGs, auxiliary winding etc. | 253 |  |  |  |  |  |  |  |
|  |  | Severe line and/or phase voltage imbalance. | 254 |  |  |  |  |  |  |  |
|  |  | Severe line current imbalance. | 255 |  |  |  |  |  |  |  |
|  |  | Under voltage. | 256 |  |  |  |  |  |  |  |
|  |  | Over voltage. | 257 |  |  |  |  |  |  |  |
|  |  | Under frequency. | 258 |  |  |  |  |  |  |  |
|  |  | Over frequency. | 259 |  |  |  |  |  |  |  |
| Power Generation | Generators | Earth fault. | 260 |  |  |  |  |  |  |  |
|  |  | Insufficient active power. | 261 |  |  |  |  |  |  |  |
|  |  | Excess active power. | 262 |  |  |  |  |  |  |  |
|  |  | Unstable active power. | 263 |  |  |  |  |  |  |  |
|  |  | Insufficient reactive power. | 264 |  |  |  |  |  |  |  |
|  |  | Excess reactive power. | 265 |  |  |  |  |  |  |  |
|  |  | Unstable reactive power. | 266 |  |  |  |  |  |  |  |
|  |  | Loss of synchronisation – pole slipping. | 267 |  |  |  |  |  |  |  |
|  |  | Harmonics to be maintained within defined limits. | 268 |  |  |  |  |  |  |  |
|  |  | Power factor. | 269 |  |  |  |  |  |  |  |
|  |  | Earthing system, NETs, NERs etc. | 270 |  |  |  |  |  |  |  |
|  |  | Hidden failure to start / connect. | 271 |  |  |  |  |  |  |  |
| Power Generation | Generators | Hidden failure insufficient capacity kW & kVAr. | 272 |  |  |  |  |  |  |  |
|  |  | Hidden failure poor load acceptance & rejection. | 273 |  |  |  |  |  |  |  |
|  |  | Description of switch. gear HV / LV arc proof, bus bar insulation type, gas insulated etc. | 274 |  |  |  |  |  |  |  |
|  | Generator Switchgear | Arc detection – optical, pressure – potential vulnerability to common mode failures. | 275 |  |  |  |  |  |  |  |
|  |  | Power management interface. | 276 |  |  |  |  |  |  |  |
|  |  | Temperature control and HVAC. | 277 |  |  |  |  |  |  |  |
|  |  | Synchronisation and crash sync. | 278 |  |  |  |  |  |  |  |
|  |  | Dead bus connection. | 279 |  |  |  |  |  |  |  |
|  |  | Interlocks – electrical and mechanical. | 280 |  |  |  |  |  |  |  |
|  |  | Manual controls & synchronising. | 281 |  |  |  |  |  |  |  |
| Power Generation | Generator Switchgear | Connection of stopped generator. | 282 |  |  |  |  |  |  |  |
|  |  | Connections crossing boundaries between switchboards for status, power, sync VT signals etc. A60 & WT for DP class 3 | 283 |  |  |  |  |  |  |  |
|  |  | Hidden failure to close / connect (one or more poles). | 284 |  |  |  |  |  |  |  |
|  |  | Hidden failure to trip on fault. | 285 |  |  |  |  |  |  |  |
|  |  | Hidden failure to open on PMS order. | 286 |  |  |  |  |  |  |  |
|  | HVAC and Chilled Water Systems | Document reference with revision numbers | 287 |  |  |  |  |  |  |  |
|  |  | Location for DP Class 3. | 288 |  |  |  |  |  |  |  |
|  |  | Description of HVAC / Chilled water systems and redundancy concept. | 289 |  |  |  |  |  |  |  |
|  |  | Configuration for DP. | 290 |  |  |  |  |  |  |  |
|  |  | Analysis identifies redundant groups in HVAC / chilled water system. | 291 |  |  |  |  |  |  |  |
| Power Generation | HVAC and Chilled Water Systems | Analysis identifies common points between redundant groups. | 292 |  |  |  |  |  |  |  |
|  |  | Analysis identifies all failure effects that can propagate from one redundant group to the other. | 293 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any protective functions upon which redundancy depends to prevent failure effects propagating. | 294 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any potential hidden failure that could defeat redundancy, including any alarms or standby redundancy. | 295 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any internal common cause failures and the barriers employed to mitigate them. | 296 |  |  |  |  |  |  |  |
| Power Generation | HVAC and Chilled Water Systems | Analysis identifies any external common cause failure and the barriers used to mitigate them. e.g. cement dust in all air intakes causing F&G to detect smoke and initiate ESD, Gas ingestion etc. | 297 |  |  |  |  |  |  |  |
|  |  | The analysis identifies any configuration errors that could defeat redundancy and any barrier to prevent this. | 298 |  |  |  |  |  |  |  |
|  |  | There is a system drawing that identifies all DP related components, the redundant groups to which they belong and, and their interfaces to other systems. | 299 |  |  |  |  |  |  |  |
|  |  | The worst case failure for those systems is clearly stated in relation to its effects on generators, thrusters, position and heading. | 300 |  |  |  |  |  |  |  |
| Power Generation | HVAC and Chilled Water Systems | There is a link to the proving trials program that confirms the findings of the FMEA. | 301 |  |  |  |  |  |  |  |
|  |  | Protection against damage for passive components where exempt from failure analysis as part of redundancy concept for DP Class 2. | 302 |  |  |  |  |  |  |  |
|  |  | A60 separation of redundant groups. | 303 |  |  |  |  |  |  |  |
|  |  | Watertight separation of redundant groups. | 304 |  |  |  |  |  |  |  |
|  |  | A60 separation of pipework for redundant groups for DP Class 3. | 305 |  |  |  |  |  |  |  |
|  |  | Bulkhead valves either side for DP Class 3. | 306 |  |  |  |  |  |  |  |
|  |  | Fan Coil Units. | 307 |  |  |  |  |  |  |  |
|  |  | Backup package air conditioners and supplies. | 308 |  |  |  |  |  |  |  |
| Power Generation | HVAC and Chilled Water Systems | Supports time to terminate if non-redundant. | 309 |  |  |  |  |  |  |  |
|  |  | Condensation – failure effects. | 310 |  |  |  |  |  |  |  |
|  | Generator Protection | Document reference. | 311 |  |  |  |  |  |  |  |
|  |  | Discussion of how protection scheme philosophy supports redundancy concept. | 312 |  |  |  |  |  |  |  |
|  |  | Switchgear - circuit breakers and contactors. | 313 |  |  |  |  |  |  |  |
|  |  | Differential protection. | 314 |  |  |  |  |  |  |  |
|  |  | Over current protection. | 315 |  |  |  |  |  |  |  |
|  |  | Over/under excitation. | 316 |  |  |  |  |  |  |  |
|  |  | Over/under voltage protection. | 317 |  |  |  |  |  |  |  |
| Power Generation | Generator Protection | Over/under frequency. | 318 |  |  |  |  |  |  |  |
|  |  | Reverse power protection. | 319 |  |  |  |  |  |  |  |
|  |  | Negative sequence protection. | 320 |  |  |  |  |  |  |  |
|  |  | Temperature alarms – shutdowns. | 321 |  |  |  |  |  |  |  |
|  |  | Lube oil pressure alarms – shutdowns. | 322 |  |  |  |  |  |  |  |
|  |  | Earth fault protection - zero sequence protection. | 323 |  |  |  |  |  |  |  |
|  |  | UPSs 110Vdc & 24Vdc - rating, type, split diode cross connections. | 324 |  |  |  |  |  |  |  |
|  |  | kW imbalance protection & alarm. | 325 |  |  |  |  |  |  |  |
|  |  | kVAr imbalance protection & alarm. | 326 |  |  |  |  |  |  |  |
|  |  | Arc detection. | 327 |  |  |  |  |  |  |  |
|  |  | Directional earth fault protection. | 328 |  |  |  |  |  |  |  |
|  |  | Advanced protection for generators. | 329 |  |  |  |  |  |  |  |
| Power Generation | Generator Protection | Busties pre-tripped by generator faults. | 330 |  |  |  |  |  |  |  |
|  |  | Connections between generator protection such as VT and broken delta signals – effect of failure. | 331 |  |  |  |  |  |  |  |
|  |  | Failure to excess or full fuel considered. | 332 |  |  |  |  |  |  |  |
|  |  | Failure to zero or insufficient fuel. | 333 |  |  |  |  |  |  |  |
|  |  | Asymmetric load sharing and failure modes. | 334 |  |  |  |  |  |  |  |
|  | Governors | Document reference. | 335 |  |  |  |  |  |  |  |
|  |  | Isochronous/droop. | 336 |  |  |  |  |  |  |  |
|  |  | Speed/load control. | 337 |  |  |  |  |  |  |  |
|  |  | PMS connections raise/lower sticking etc. | 338 |  |  |  |  |  |  |  |
|  |  | Overload protection - load reduction function. | 339 |  |  |  |  |  |  |  |
|  |  | Mechanical backup governor. | 340 |  |  |  |  |  |  |  |
| Power Generation | Governors | Rack position indication. | 341 |  |  |  |  |  |  |  |
|  |  | Sharing lines. | 342 |  |  |  |  |  |  |  |
|  |  | Default to droop on sharing line failure. | 343 |  |  |  |  |  |  |  |
|  | Voltage Dip | Ride through capability at all levels. | 344 |  | * |  |  |  |  |  |
|  |  | Effect on system voltage, clearing SC faults. | 345 |  | * |  |  |  |  |  |
|  |  | Effect of starting heavy consumers. | 346 |  |  |  |  |  |  |  |
|  |  | Effect on contactors for pumps etc. | 347 |  |  |  |  |  |  |  |
|  |  | Effect on power electronic drives – settings. | 348 |  |  |  |  |  |  |  |
|  |  | Effect on instrumentation and control systems. | 349 |  |  |  |  |  |  |  |
|  | AVR | Document reference. | 350 |  |  |  |  |  |  |  |
|  |  | Type of regulator and exciter. | 351 |  |  |  |  |  |  |  |
|  |  | Droop characteristics. | 352 |  |  |  |  |  |  |  |
| Power Generation | AVR | Failure mode - to over / under excitation. | 353 |  |  |  |  |  |  |  |
|  |  | Reactive power sharing. | 354 |  |  |  |  |  |  |  |
|  |  | Control by power management raise / lower. | 355 |  |  |  |  |  |  |  |
|  |  | Excitation limiters – effect on selectivity. | 356 |  |  |  |  |  |  |  |
|  |  | Field failure protection. | 357 |  |  |  |  |  |  |  |
|  |  | Interaction with thrust limitation. | 358 |  |  |  |  |  |  |  |
|  |  | Hidden failure - Current boost systems. | 359 |  |  |  |  |  |  |  |
|  |  | Hidden failure - Excitation support. | 360 |  |  |  |  |  |  |  |
|  |  | Hidden failure – fault or not specified PMG. | 361 |  |  |  |  |  |  |  |
|  |  | Cross current compensation. | 362 |  |  |  |  |  |  |  |
|  |  | Interface with protection for alternator – differential protection. | 363 |  |  |  |  |  |  |  |
| Power Generation | AVR | Interface with HV switchgear – earth switch. | 364 |  |  |  |  |  |  |  |
| Power Management | Power Management Scope | Document reference. | 365 |  |  |  |  |  |  |  |
|  |  | Philosophy - Continuity of electrical power, Ability to restore propulsion capability Ability to restore propulsion capability. | 366 |  |  |  |  |  |  |  |
|  |  | Security and failure modes of ‘Power Available’ calculator. | 367 |  |  |  |  |  |  |  |
|  |  | Topology of PMS hardware – distributed system or centralised. | 368 |  |  |  |  |  |  |  |
|  |  | PMS functions considered as potential hidden failures. | 369 |  |  |  |  |  |  |  |
|  |  | Diesel automation. | 370 |  |  |  |  |  |  |  |
|  |  | Control of engine starting requirements. | 371 |  |  |  |  |  |  |  |
| Power Management | Power Management Scope | Control of engine specific requirements. | 372 |  |  |  |  |  |  |  |
|  |  | Selection of generator running order. | 373 |  |  |  |  |  |  |  |
|  |  | Load dependent starting of generators on kW, kVAr, Amps? – Appropriate. | 374 |  |  |  |  |  |  |  |
|  |  | Load dependent stop – inhibited on DP. | 375 |  |  |  |  |  |  |  |
|  |  | Alarm start – but no shutdown. | 376 |  |  |  |  |  |  |  |
|  |  | Blackout prevention system – Philosophy integrated with drives, industrial consumers, DP control systems. | 377 |  |  |  |  |  |  |  |
|  |  | Auto reconfiguration of circuit breakers. | 378 |  |  |  |  |  |  |  |
|  |  | Advanced power reservation – start blocking. | 379 |  |  |  |  |  |  |  |
|  |  | Interface with electronic governor. | 380 |  |  |  |  |  |  |  |
|  |  | Interface with AVR. | 381 |  |  |  |  |  |  |  |
| Power Management | Power Management Scope | Interface with engine management system. | 382 |  |  |  |  |  |  |  |
|  |  | Active power sharing. | 383 |  |  |  |  |  |  |  |
|  |  | Reactive power sharing. | 384 |  |  |  |  |  |  |  |
|  |  | Net frequency control. | 385 |  |  |  |  |  |  |  |
|  |  | Asymmetric load sharing. | 386 |  |  |  |  |  |  |  |
|  |  | Protective functions in PMS – Bespoke. | 387 |  |  |  |  |  |  |  |
|  |  | Maintain harmonics within limits. | 388 |  |  |  |  |  |  |  |
|  |  | Control of switchboard fault levels –e.g. open busties on connection of last generator. | 389 |  |  |  |  |  |  |  |
|  |  | Control of regenerated power from draw works etc. prevent generators tripping on reverse power. | 390 |  |  |  |  |  |  |  |
|  |  | Reference to Vendor FMEA. | 391 |  |  |  |  |  |  |  |
| Power Management | Transducers & Status | Bus tie status. | 392 |  |  |  |  |  |  |  |
|  |  | Generator circuit breaker status. | 393 |  |  |  |  |  |  |  |
|  |  | Transducer's power supplies and failure modes. | 394 |  |  |  |  |  |  |  |
|  |  | Dead bus and blackout detection – security of blackout protection effect of spurious detection may create real blackout. | 395 |  |  |  |  |  |  |  |
|  | Architecture | Hierarchy of control systems – local, PMS, DP control. | 396 |  |  |  |  |  |  |  |
|  |  | Redundancy concept centralised or distributed. | 397 |  |  |  |  |  |  |  |
|  |  | System independence and segregation. | 398 |  |  |  |  |  |  |  |
|  |  | Autonomy of main machinery. | 399 |  |  |  |  |  |  |  |
|  |  | Common points between redundant systems. | 400 |  |  |  |  |  |  |  |
| Power Management | Architecture | Common mode failures. | 401 |  |  |  |  |  |  |  |
|  |  | Hidden failures. | 402 |  |  |  |  |  |  |  |
|  |  | Barriers to operator error – maloperation. | 403 |  |  |  |  |  |  |  |
|  |  | Data communications networks. | 404 |  |  |  |  |  |  |  |
|  |  | Analogue I/O distribution across redundant groups – particular for DP Class 3, effects of fire and flooding on same. | 405 |  |  |  |  |  |  |  |
|  | Synchronising | Control of dedicated synchronisers. | 406 |  |  |  |  |  |  |  |
|  |  | Voltage control. | 407 |  |  |  |  |  |  |  |
|  |  | Frequency control. | 408 |  |  |  |  |  |  |  |
|  |  | Reduce power transfer across bus tie. | 409 |  |  |  |  |  |  |  |
|  |  | Crash sync of generators and switchboards. | 410 |  |  |  |  |  |  |  |
| Power Management | Synchronising | Connection of a stopped generator. | 411 |  |  |  |  |  |  |  |
|  |  | Loss of sync due to mechanical failure. | 412 |  |  |  |  |  |  |  |
|  | Load Sharing | Speed droop. | 413 |  |  |  |  |  |  |  |
|  |  | Compensated droop. | 414 |  |  |  |  |  |  |  |
|  | Blackout Prevention | Load dependent start. | 415 |  |  |  |  |  |  |  |
|  |  | Start blocking. | 416 |  |  |  |  |  |  |  |
|  |  | Heavy consumer protection. | 417 |  |  |  |  |  |  |  |
|  |  | Load limitation/shedding. | 418 |  |  |  |  |  |  |  |
|  |  | Thrust limitation/reduction. | 419 |  |  |  |  |  |  |  |
|  |  | Thruster tripping. | 420 |  |  |  |  |  |  |  |
|  |  | Preferential tripping. | 421 |  |  |  |  |  |  |  |
|  |  | Interfaces with other blackout protection. | 422 |  |  |  |  |  |  |  |
|  |  | Other systems interaction. | 423 |  |  |  |  |  |  |  |
| Power Management | Blackout Prevention | Thruster priority. | 424 |  |  |  |  |  |  |  |
|  |  | Heading priority. | 425 |  |  |  |  |  |  |  |
|  |  | Process/drilling priority. | 426 |  |  |  |  |  |  |  |
|  |  | Process phase back. | 427 |  |  |  |  |  |  |  |
|  |  | Analysis considers - Effect of large thruster driving off initiating phase back on other thrusters. | 428 |  |  |  |  |  |  |  |
|  |  | Effect of governor failure to full fuel initiating phase back leading to loss of position and/or blackout. | 429 |  |  |  |  |  |  |  |
|  | Blackout Restart | Generator starting sequence or start all simultaneously. | 430 |  |  |  |  |  |  |  |
|  |  | Dead bus connection. | 431 |  |  |  |  |  |  |  |
|  |  | Protection against faulty generator connection. | 432 |  |  |  |  |  |  |  |
| Power Management | Blackout Restart | Protection against simultaneous connection of generators. | 433 |  |  |  |  |  |  |  |
|  |  | Staged reconnection of loads. | 434 |  |  |  |  |  |  |  |
|  |  | Lock outs of healthy generators – unnecessary – prevents reconnection of healthy generators. | 435 |  |  |  |  |  |  |  |
|  | Static Power Estimate | Configurations for DP CAM & TAM – Note CAM previously referred to as SMO – Safest Mode of Operation, in previous revisions. | 436 |  |  |  |  |  |  |  |
|  |  | Load and thrust levels that provide fault tolerance. | 437 |  |  |  |  |  |  |  |
|  |  | Calculations consider active power, reactive power, current and power factor, load acceptance and rejection. | 438 |  |  |  |  |  |  |  |
| Power Management | Interfaces | Drilling PMS - Interfaces – failure modes – hidden failures – means to reveal loss of data link analogue connections etc. Modes for’ locked to bottom’, Drilling priority and means to dump it, staged phase back of mud pumps etc., reserved power margin for active heave compensation. – landing BOP etc. | 439 |  |  |  |  |  |  |  |
|  |  | Crane – Phased back before thrusters. | 440 |  |  |  |  |  |  |  |
|  |  | Pipe lay systems – Phased back before thrusters – reserved power margin. | 441 |  |  |  |  |  |  |  |
|  |  | Other industrial mission equipment. | 442 |  |  |  |  |  |  |  |
| Power Distribution | Concept and Philosophy | Document reference with revision numbers. | 443 |  |  |  |  |  |  |  |
|  |  | Description of power distribution systems and redundancy concept. | 444 |  |  |  |  |  |  |  |
|  |  | Type and location of all switchboards DBs etc. for DP Class 3. | 445 |  |  |  |  |  |  |  |
|  |  | Type and location of all service and industrial mission transformers – cooling etc. | 446 |  |  |  |  |  |  |  |
|  |  | Configuration for DP CAM & TAM etc. | 447 |  |  |  |  |  |  |  |
|  |  | Analysis identifies redundant power systems and confirms they support surge sway and yaw in intact and post WCF. | 448 |  |  |  |  |  |  |  |
|  |  | Analysis identifies common points between redundant power systems. | 449 |  |  |  |  |  |  |  |
| Power Distribution | Concept and Philosophy | Analysis identifies all failure effects that can propagate from one redundant group to the other. | 450 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any protective functions upon which redundancy depends to prevent failure effects propagating. | 451 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any potential hidden failure that could defeat redundancy, including any alarms or standby redundancy. | 452 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any internal common cause failures and the barriers employed to mitigate them. | 453 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any external common cause failure and the barriers used to mitigate them. | 454 |  |  |  |  |  |  |  |
| Power Distribution | Concept and Philosophy | The analysis identifies any configuration errors that could defeat redundancy and any barrier to prevent this. | 455 |  |  |  |  |  |  |  |
|  |  | There is a system drawing that identifies all DP related components, the redundant groups to which they belong and, and their interfaces to other systems. | 456 |  |  |  |  |  |  |  |
|  |  | The worst case failure for those systems is clearly stated in relation to its effects on generators, thrusters, position and heading. | 457 |  |  |  |  |  |  |  |
|  |  | There is a link to the proving trials program that confirms the findings of the FMEA. | 458 |  |  |  |  |  |  |  |
|  |  | Type of power system – three wire, three phase for example. | 459 |  |  |  |  |  |  |  |
| Power Distribution | Concept and Philosophy | Single bustie switchboards – analysis of potential for unscheduled closing. | 460 |  |  |  |  |  |  |  |
|  |  | Master slave busties – potential for both to close together – leading to crash sync | 461 |  |  |  |  |  |  |  |
|  |  | Protection against damage for passive components where exempt from failure analysis as part of redundancy concept for DP Class 2. | 462 |  |  |  |  |  |  |  |
|  |  | A60 separation of redundant groups. | 463 |  |  |  |  |  |  |  |
|  |  | Watertight separation of redundant groups. | 464 |  |  |  |  |  |  |  |
|  | Studies | Protection philosophy. | 465 |  |  |  |  |  |  |  |
|  |  | Reference to load balance. | 466 |  |  |  |  |  |  |  |
|  |  | Reference to selectivity / coordination study | 467 |  |  |  |  |  |  |  |
| Power Distribution | Studies | Reference to harmonics study in intact and post WCF of harmonic control features | 468 |  |  |  |  |  |  |  |
|  |  | Reference to transient stability study showing effects of WC short circuit, effects of crash sync of generators and switchboard, connection of a stopped generator | 469 |  |  |  |  |  |  |  |
|  |  | Reference to short circuit and earth fault test of main switchboards on DP. | 470 |  |  |  |  |  |  |  |
|  | Interlocks and Protection | Interlocks and intertrips – crossing redundant groups | 471 |  |  |  |  |  |  |  |
|  |  | Short circuit protection | 472 |  |  |  |  |  |  |  |
|  |  | Earth fault protection | 473 |  |  |  |  |  |  |  |
|  |  | Motor starting and protection | 474 |  |  |  |  |  |  |  |
| Power Distribution | Concept and Philosophy | List MCCs and distribution boards & tables of DP related consumers. | 475 |  |  |  |  |  |  |  |
|  |  | Power for switchboard controls. | 476 |  |  |  |  |  |  |  |
|  |  | Power for spring winders from where? | 477 |  |  |  |  |  |  |  |
|  |  | UPSs for VMS and DP control systems- analysis confirm the type of UPS e.g. double conversion or line interactive type. If line interactive the analysis investigates the potential for switching transients to pass through this type of UPS. Bypass arrangements reviewed and alarms for ‘bypass selected’ – normal and alternative supplies. Means to prevent transfer of faults. Alarms for on-batteries and battery disconnected confirmed. | 478 |  |  |  |  |  |  |  |
| Power Distribution | Concept and Philosophy | Maintaining DP ready signal during switching of transferable thrusters. | 479 |  |  |  |  |  |  |  |
|  |  | Transformer cooling fan supplies. | 480 |  |  |  |  |  |  |  |
|  |  | Emergency generator starting. | 481 |  |  |  |  |  |  |  |
|  |  | Shore power interlocks and inter trips. | 482 |  |  |  |  |  |  |  |
|  |  | Back feeding from emergency generator. | 483 |  |  |  |  |  |  |  |
|  |  | Protection against primary to secondary faults in service transformers. | 484 |  |  |  |  |  |  |  |
|  |  | Pre-magnetising systems – effects of failure to operate, effects of spurious operation. | 485 |  |  |  |  |  |  |  |
|  |  | Consumers on deck fed from DP switchboards. | 486 |  |  |  |  |  |  |  |
| Power Distribution | Safety Systems | Interface to ESD. | 487 |  |  |  |  |  |  |  |
|  |  | Interface to group emergency stops. | 488 |  |  |  |  |  |  |  |
|  |  | Interface to F&G, Water mist etc. | 489 |  |  |  |  |  |  |  |
|  | Concept and Philosophy | Commonality created by colocation of industrial consumers for DP Class 3 – effects of fire and flooding – voltage dips on all redundant power systems for > WCFDI. | 490 |  |  |  |  |  |  |  |
|  |  | Effects of fire and flooding on protection coordination – multiple fault paths – sequential voltage dips. | 491 |  |  |  |  |  |  |  |
|  |  | Cable routes for DP Class 3 and high risk areas for DP Class 2. | 492 |  |  |  |  |  |  |  |
| Propellers and Steering Gear | Main Propellers | Role of main propellers in redundancy concept – system drawings showing all interfaces and services upon which the propulsion train depends. | 493 |  |  |  |  |  |  |  |
|  |  | Drawing reference. | 494 |  |  |  |  |  |  |  |
|  |  | Gearbox – common to more than one redundant group – shaft generators on PTO etc. | 495 |  |  |  |  |  |  |  |
|  |  | Clutch – Failure modes on loss of power and signal. | 496 |  |  |  |  |  |  |  |
|  |  | CPP – Failure modes on loss of power and signal command / feedback. | 497 |  |  |  |  |  |  |  |
|  |  | Speed control – failure modes on loss of command / feedback – signal and ground – earth fault / short circuit / open circuit. | 498 |  |  |  |  |  |  |  |
|  |  | Interaction of variable speed control. | 499 |  |  |  |  |  |  |  |
| Propellers and Steering Gear | Main Propellers | Power supplies, main, auxiliary and backup. | 500 |  |  |  |  |  |  |  |
|  |  | Pumps from main supply, from emergency supplies. | 501 |  |  |  |  |  |  |  |
|  |  | Hydraulics – Effect of loss of pressure (loss of ready?) | 502 |  |  |  |  |  |  |  |
|  |  | Lube oil – Can prop run without forced circulation. | 503 |  |  |  |  |  |  |  |
|  |  | Cooling as part of marine auxiliary systems for other thrusters? Or shared with service transformers in some cases – Failure effects. | 504 |  |  |  |  |  |  |  |
|  |  | Operating modes. | 505 |  |  |  |  |  |  |  |
|  |  | Failure to ahead condition for main Class. | 506 |  |  |  |  |  |  |  |
|  |  | Control stations, alternative controls. | 507 |  |  |  |  |  |  |  |
| Propellers and Steering Gear | Steering Gear | Control power supplies main/alternative. | 508 |  |  |  |  |  |  |  |
|  |  | DP ready signal requirements. | 509 |  |  |  |  |  |  |  |
|  |  | Alarms. | 510 |  |  |  |  |  |  |  |
|  |  | Control loop protection. | 511 |  |  |  |  |  |  |  |
|  |  | Interface with rudder. | 512 |  |  |  |  |  |  |  |
|  |  | Drawing reference. | 513 |  |  |  |  |  |  |  |
|  |  | Power supplies, main, auxiliary and backup – commonality introduced by using emergency switchboard. | 514 |  |  |  |  |  |  |  |
|  |  | Pumps. | 515 |  |  |  |  |  |  |  |
|  |  | Hydraulics. | 516 |  |  |  |  |  |  |  |
|  |  | Cooling. | 517 |  |  |  |  |  |  |  |
|  |  | Operating modes. | 518 |  |  |  |  |  |  |  |
| Propellers and Steering Gear | Steering Gear | Defaults, especially on loss of propulsion. | 519 |  |  |  |  |  |  |  |
|  |  | Control stations, alternative controls. | 520 |  |  |  |  |  |  |  |
|  |  | Alternative controls. | 521 |  |  |  |  |  |  |  |
|  |  | Emergency controls. | 522 |  |  |  |  |  |  |  |
|  |  | Control power supplies. | 523 |  |  |  |  |  |  |  |
|  |  | Ready signal requirements. | 524 |  |  |  |  |  |  |  |
|  |  | Control loop protection. | 525 |  |  |  |  |  |  |  |
|  |  | Angle indication. | 526 |  |  |  |  |  |  |  |
|  |  | Mechanically linked rudders on twin screw vessels. | 527 |  |  |  |  |  |  |  |
|  |  | Failure modes including to 'hard over'. | 528 |  |  |  |  |  |  |  |
|  |  | High lift rudders for DP. | 529 |  |  |  |  |  |  |  |
|  |  | Alarms. | 530 |  |  |  |  |  |  |  |
| Propellers and Steering Gear | Tunnel and Azimuth Thrusters | Drawing reference. | 531 |  |  |  |  |  |  |  |
|  |  | Location – drawing? – Plan view showing thruster locations and association with each redundant machinery group. | 532 |  |  |  |  |  |  |  |
|  |  | Type FPP, CPP, Tunnel, Azimuth, Combinator, Voith Schneider, etc. | 533 |  |  |  |  |  |  |  |
|  |  | Specification – Thrust / Power curve presented. | 534 |  |  |  |  |  |  |  |
|  |  | Prime movers – Motors, diesel engines? Interlocks and start / run permissive – any commonality introduced by this discussed? | 535 |  |  |  |  |  |  |  |
|  |  | Main power – Interlocks – drive controls CB? Estop independent of thruster control system? | 536 |  |  |  |  |  |  |  |
| Propellers and Steering Gear | Tunnel and Azimuth Thrusters | System drawings showing all thruster auxiliary services and their interfaces to other systems. | 537 |  |  |  |  |  |  |  |
|  |  | Gearbox- services, lubrication, sealing air pressure | 538 |  |  |  |  |  |  |  |
|  |  | Clutch – Failure modes on loss of power and signal open? Slipping clutch for direct drive, pulsed clutch? | 539 |  |  |  |  |  |  |  |
|  |  | CPP- Failure mode to full pitch on loss of hydraulic pressure? | 540 |  |  |  |  |  |  |  |
|  |  | Speed/torque control – means of transferring between the two modes – Failure modes and effects of control loops. | 541 |  |  |  |  |  |  |  |
|  |  | Interaction of variable speed control – Does thrust drop while thruster is rotating in azimuth? | 542 |  |  |  |  |  |  |  |
| Propellers and Steering Gear | Tunnel and Azimuth Thrusters | Power supplies, main, auxiliary and backup | 543 |  |  |  |  |  |  |  |
|  |  | Pumps – duty standby? – What triggers C/O? | 544 |  |  |  |  |  |  |  |
|  |  | Hydraulics – effect of loss of pressure? | 545 |  |  |  |  |  |  |  |
|  |  | Lube oil – Operation without forced lubrication? | 546 |  |  |  |  |  |  |  |
|  |  | Cooling – commonality with other systems? | 547 |  |  |  |  |  |  |  |
|  |  | Control stations, alternative controls on field stations etc. | 548 |  |  |  |  |  |  |  |
|  |  | Control power supplies main/alternative | 549 |  |  |  |  |  |  |  |
|  |  | DP ready signal requirements | 550 |  |  |  |  |  |  |  |
|  |  | Barred zones – Does analysis confirm zones deactivate when thruster is deselected? | 551 |  |  |  |  |  |  |  |
| Propellers and Steering Gear | Tunnel and Azimuth Thrusters | Mode selection switches – NFU controls – effects of fire and flooding on same for DP Class 3? | 552 |  |  |  |  |  |  |  |
|  |  | Rate of turning – 2 RPM? | 553 |  |  |  |  |  |  |  |
|  |  | Ramp rate 0 to 100% thrust < 30s. | 554 |  |  |  |  |  |  |  |
|  |  | Thruster control modes. | 555 |  |  |  |  |  |  |  |
|  | Azimuth Thrusters | Drawing reference. | 556 |  |  |  |  |  |  |  |
|  |  | Power supplies, main, auxiliary and backup. | 557 |  |  |  |  |  |  |  |
|  |  | Pumps. | 558 |  |  |  |  |  |  |  |
|  |  | Hydraulics. | 559 |  |  |  |  |  |  |  |
|  |  | Cooling. | 560 |  |  |  |  |  |  |  |
|  |  | Control stations. | 561 |  |  |  |  |  |  |  |
|  |  | Alternative controls. | 562 |  |  |  |  |  |  |  |
| Propellers and Steering Gear | Azimuth Thrusters | Emergency controls. | 563 |  |  |  |  |  |  |  |
|  |  | Control power supplies. | 564 |  |  |  |  |  |  |  |
|  |  | DP ready signal requirements. | 565 |  |  |  |  |  |  |  |
|  |  | Control loop protection. | 566 |  |  |  |  |  |  |  |
|  |  | Angle indication. | 567 |  |  |  |  |  |  |  |
|  |  | Alarms. | 568 |  |  |  |  |  |  |  |
|  |  | Azimuth control loops – Failure modes. | 569 |  |  |  |  |  |  |  |
| Vessel Management System | General | Drawing reference. | 570 |  |  |  |  |  |  |  |
|  |  | General description. | 571 |  |  |  |  |  |  |  |
|  |  | System architecture. | 572 |  |  |  |  |  |  |  |
|  |  | Redundancy concept. | 573 |  |  |  |  |  |  |  |
|  |  | System segregation. | 574 |  |  |  |  |  |  |  |
|  |  | System independence and duplication. | 575 |  |  |  |  |  |  |  |
| Vessel Management System | General | Switching and protective functions. | 576 |  |  |  |  |  |  |  |
|  |  | Common mode failures. | 577 |  |  |  |  |  |  |  |
|  |  | Hidden failures. | 578 |  |  |  |  |  |  |  |
|  |  | Multiple independent failures. | 579 |  |  |  |  |  |  |  |
|  |  | Operator error. | 580 |  |  |  |  |  |  |  |
|  | Switchboard PLC Comms | Reaction of engines to PLC failures. | 581 |  |  |  |  |  |  |  |
|  | Air Conditioning | Component susceptibility to overheating. | 582 |  |  |  |  |  |  |  |
|  |  | Provision of A/C redundancy. | 583 |  |  |  |  |  |  |  |
|  |  | Alarms for temperature increase / A/C failure. | 584 |  |  |  |  |  |  |  |
|  | Network Topology and Redundancy Concept | Document reference with revision numbers. | 585 |  |  |  |  |  |  |  |
|  |  | Description of vessel management system and redundancy concept. | 586 |  |  |  |  |  |  |  |
| Vessel Management System | Network Topology and Redundancy Concept | Type and location of all switchboards DBs etc. for DP Class 3. | 587 |  |  |  |  |  |  |  |
|  |  | Thruster control modes – bias, fixed, variable. | 588 |  |  |  |  |  |  |  |
|  |  | Type and location of all hubs, switches, field stations, operator stations etc. for DP Class 3. | 589 |  |  |  |  |  |  |  |
|  |  | Analysis identifies assignment of field stations and their UPS power supplies to thrusters and confirms they support surge sway and yaw in intact and post WCF. | 590 |  |  |  |  |  |  |  |
|  |  | Analysis identifies assignment of field stations and their UPS power supplies generators and switchboards and confirms the arrangement support the WCFDI. | 591 |  |  |  |  |  |  |  |
| Vessel Management System | Network Topology and Redundancy Concept | Analysis identifies assignment of field stations and their UPS power supplies to marine auxiliary systems and confirms it introduces no unacceptable commonality. | 592 |  |  |  |  |  |  |  |
|  |  | Analysis identifies assignment of field stations and their UPS power supplies to ESD and F&G systems and confirms it introduces no unacceptable commonality. | 593 |  |  |  |  |  |  |  |
|  |  | Analysis identifies common points between redundant power systems? | 594 |  |  |  |  |  |  |  |
|  |  | Analysis identifies all failure effects that can propagate from one redundant group to the other. | 595 |  |  |  |  |  |  |  |
| Vessel Management System | Network Topology and Redundancy Concept | Analysis identifies any protective functions upon which redundancy depends to prevent failure effects propagating. | 596 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any potential hidden failure that could defeat redundancy, including any alarms or standby redundancy. | 597 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any internal common cause failures and the barriers employed to mitigate them. | 598 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any external common cause failure and the barriers used to mitigate them. | 599 |  |  |  |  |  |  |  |
|  |  | The analysis identifies any configuration errors that could defeat redundancy and any barrier to prevent this. | 600 |  |  |  |  |  |  |  |
| Vessel Management System | Network Topology and Redundancy Concept | There is a system drawing that identifies all DP related components, the redundant groups to which they belong and, and their interfaces to other systems. | 601 |  |  |  |  |  |  |  |
|  |  | The worst case failure for this system is clearly stated in relation to its effects on generators, thrusters, position and heading. | 602 |  |  |  |  |  |  |  |
|  |  | There is a link to the proving trials program that confirms the findings of the FMEA. | 603 |  |  |  |  |  |  |  |
|  |  | Common mode failures – Network storms. | 604 |  |  |  |  |  |  |  |
|  |  | Capacity sufficient for intact and post WCF. | 605 |  |  |  |  |  |  |  |
|  |  | Alarm to indicate hidden failures – Field stations, Ethernet cables, hubs, failure of one dual power supply. | 606 |  |  |  |  |  |  |  |
| Vessel Management System | Network Topology and Redundancy Concept | Virus protection. | 607 |  |  |  |  |  |  |  |
|  |  | Scan rates. | 608 |  |  |  |  |  |  |  |
|  |  | Remote access for diagnostics. | 609 |  |  |  |  |  |  |  |
|  |  | Reference to VMS vendor FMEA. | 610 |  |  |  |  |  |  |  |
| DP Control Systems | Concept | Document reference with revision numbers. | 611 |  |  |  |  |  |  |  |
|  |  | Reference to DP control system vendor FMEA. | 612 |  |  |  |  |  |  |  |
|  |  | Location for DP Class 3. | 613 |  |  |  |  |  |  |  |
|  |  | Manufacturer, type and model number of DP control system- duplex, triplex etc. | 614 |  |  |  |  |  |  |  |
|  |  | Description of DP control systems and redundancy concept. | 615 |  |  |  |  |  |  |  |
|  |  | Configuration for DP. | 616 |  |  |  |  |  |  |  |
|  |  | Analysis identifies redundant groups in DP control system. | 617 |  |  |  |  |  |  |  |
| DP Control Systems | Concept | Analysis identifies common points between redundant groups. | 618 |  |  |  |  |  |  |  |
|  |  | Analysis identifies all failure effects that can propagate from one redundant group to the other. | 619 |  |  |  |  |  |  |  |
|  |  | Analysis identified any protective functions upon which redundancy depends to prevent failure effects propagating. | 620 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any potential hidden failure that could defeat redundancy, including any alarms or standby redundancy. | 621 |  |  |  |  |  |  |  |
|  |  | Analysis identifies any internal common cause failures and the barriers employed to mitigate them. | 622 |  |  |  |  |  |  |  |
| DP Control Systems | Concept | Analysis identifies any external common cause failure and the barriers used to mitigate them. | 623 |  |  |  |  |  |  |  |
|  |  | The analysis identifies any configuration errors that could defeat redundancy and any barrier to prevent this | 624 |  |  |  |  |  |  |  |
|  |  | There is a system drawing that identifies all DP related components, the redundant groups to which they belong and, and their interfaces to other systems. | 625 |  |  |  |  |  |  |  |
|  |  | The worst case failure for the DP control system is clearly stated in relation to its effects on generators, thrusters, position and heading. | 626 |  |  |  |  |  |  |  |
|  |  | There is a link to the proving trials program that confirms the findings of the FMEA. | 627 |  |  |  |  |  |  |  |
| DP Control Systems | Concept | In DP control systems with an analogue interface to the thrusters the provision of interface modules / IO cards matches the overall split in the redundancy concept. | 628 |  |  |  |  |  |  |  |
|  |  | Changeover switch between main and backup for DP Class 3 – all failure modes considered including effects of fire (and flooding if below damaged waterline). | 629 |  |  |  |  |  |  |  |
|  |  | Changeover switch between DP and manual, IJS etc. – Failure modes. | 630 |  |  |  |  |  |  |  |
|  |  | Analysis confirms no single failure of the mode selection switch prevents manual control. | 631 |  |  |  |  |  |  |  |
|  |  | Analysis confirms no single failure of the mode selector switch exceeds WCFDI. | 632 |  |  |  |  |  |  |  |
| DP Control Systems | Concept | Analysis discusses means to prevent inadvertent operation – double push or its successor. | 633 |  |  |  |  |  |  |  |
|  |  | Drawing reference. | 634 |  |  |  |  |  |  |  |
|  |  | Analysis confirms at least three independent position reference systems based on two different principles. | 635 |  |  |  |  |  |  |  |
|  |  | Analysis confirms the reference systems are suitable for the vessel’s industrial mission | 636 |  |  |  |  |  |  |  |
|  |  | Analysis confirms the worst case loss of reference systems leaves at least one operational (DNV), does not exceed one (ABS) | 637 |  |  |  |  |  |  |  |
|  |  | Independent joystick (IJS) – Failure modes and effect on DP system while in auto DP. | 638 |  |  |  |  |  |  |  |
| DP Control Systems | IJS | IJS – Analysis confirms independence of main DP control system, independent of VMS network, independent of DP power supply. | 639 |  |  |  |  |  |  |  |
|  | PMS | DP Thrust Limitation – discussed –failure modes effects of transducer and status contact failure – single bus philosophy for dual fed thrusters. | 640 |  |  |  |  |  |  |  |
|  | Networks | Local Networks - Commonality in local networks from interface to I/O modules for gyro ready signal, thruster, thruster command and feedback ready signal etc. RS485 susceptible to jabber. | 641 |  |  |  |  |  |  |  |
| DP Control Systems | References and Sensors | Heading – Analysis confirms the provision of three gyros – common mode failures associated with common measurement principle addressed. Perhaps by spinning mass/ FOG combo. | 642 |  |  |  |  |  |  |  |
|  |  | Motion – Analysis confirms the provision of at least 3 MRUs – failure modes include inert state and false roll, pitch or heave? – Faulty telegram etc. | 643 |  |  |  |  |  |  |  |
|  |  | Wind sensors – Failure modes to high and low wind speed and erroneous direction. Common mode failures including icing, trace heating failure, common mode failure of ultrasonic wind sensors in rain. | 644 |  |  |  |  |  |  |  |
| DP Control Systems | References and Sensors | GNSS (Satnav) – Analysis confirms that DGNSS systems are well separated in terms of power supplies, receivers for GNSS and corrections. Protective functions designed to reject or reduce the risk faulty GNSS data are discussed – Elevation masks etc. common mode failures related to radio interference, poor antenna separation, tropical rain storms, constellation jumps, use of > 2 DGNSS in DP control systems. | 645 |  |  |  |  |  |  |  |
|  |  | Hydro acoustic – Analysis confirms the independence of dual HPRs and discusses the provision of power supplies, gyro, MRU and DGNSS inputs. Potential common mode failures in respect of thruster noise and sub sea activities etc. | 646 |  |  |  |  |  |  |  |
| DP Control Systems | References and Sensors | Taut wire – Alignment of supplies for various parts of TW, TW computer etc. Effect of losing hoist power, air supply. Discussion of alarms and protective functions to reduce the risk of faulty TW data – ships side box, effects of thruster wash on TW. Barred zones analysed. | 647 |  |  |  |  |  |  |  |
|  |  | Microwave – Artemis – Provision of power supplies, range restrictions interference of older sets from radar. | 648 |  |  |  |  |  |  |  |
|  |  | Laser – Fan beam – Provision of power supplies for scanning head, control box and monitor. Discussion of short comings of laser position references, range – reflector v prism in. Locking on to false targets. Saturation of receiver in direct sunlight. | 649 |  |  |  |  |  |  |  |
| DP Control Systems | References and Sensors | Seapath – Use as heading input discussed, interface to gyro, MRUs, power supply discussed in relation to overall redundancy concept for DP control system. | 650 |  |  |  |  |  |  |  |
|  |  | Radius – Power supplies reflect split in redundancy concept. | 651 |  |  |  |  |  |  |  |
|  |  | RadaScan as for Radius. | 652 |  |  |  |  |  |  |  |
|  |  | Draught sensor – nature of input / automatic V manual – Failure modes, commonality of sensor inputs (single field station for all). Appropriate bounds on draft data. | 653 |  |  |  |  |  |  |  |
| DP Control Systems | References and Sensors | Riser angle – unlikely to be used as position reference but available to DPO as confirmation of position excursion during position reference system problems. | 654 |  |  |  |  |  |  |  |
|  |  | Tensioner stroke - available to DPO as confirmation of position excursion during position reference system problems. | 655 |  |  |  |  |  |  |  |
|  |  | Pipe tension – Manual input versus automatic. Potential for erroneous input to affect mathematical model – systems engine room approach taken to design of input. | 656 |  |  |  |  |  |  |  |
|  |  | Crane angle – Effect of failures, range of failure modes considered including erroneous angle. | 657 |  |  |  |  |  |  |  |
| DP Control Systems | References and Sensors | Survey package – Effects of failures, IMCA S009 considered | 658 |  |  |  |  |  |  |  |
|  |  | Analysis considers ‘jabber’ on serial interface from reference systems etc. casing process overload and loss of position. | 659 |  |  |  |  |  |  |  |
|  |  | Analysis discusses all protective functions associated with rejecting an erroneous reference signal. | 660 |  |  |  |  |  |  |  |
|  |  | Analysis discusses all protective functions associated with rejecting an erroneous vessel sensor signal. | 661 |  |  |  |  |  |  |  |
|  | DP Alert and Comms. | Provision of DP status lights – hard wired or over network (should operate during total systems failure). | 662 |  |  |  |  |  |  |  |
|  |  | Location of alarms and indication. | 663 |  |  |  |  |  |  |  |
| DP Control Systems | DP Alert and Comms. | Automatic functions related to position failure. | 664 |  |  |  |  |  |  |  |
|  | Cable Routes | Cable route for thruster control and feedback. | 665 |  |  |  |  |  |  |  |
|  |  | Philosophy of cable routing in respect of fire. | 666 |  |  |  |  |  |  |  |
|  |  | Cable routes to vessel sensors and reference. | 667 |  |  |  |  |  |  |  |
|  | Backup DP System | Independent and isolated from main DP system except for network connections etc. | 668 |  |  |  |  |  |  |  |
|  |  | Isolation box for shared reference systems. | 669 |  |  |  |  |  |  |  |
|  |  | Backup updated from main DP system. | 670 |  |  |  |  |  |  |  |
|  |  | Backup should have separate climate control. | 671 |  |  |  |  |  |  |  |
| Emergency Shut Downs | ESDs | Drawing reference. | 672 |  |  |  |  |  |  |  |
|  |  | I/O and field stations Split along lines of redundancy concept. | 673 |  |  |  |  |  |  |  |
|  |  | Lower level shutdowns (below total) match redundancy concept so that one redundant machinery group can be shutdown at a time without loss of position. | 674 |  |  |  |  |  |  |  |
|  |  | Effect on DP. | 675 |  |  |  |  |  |  |  |
|  |  | Line monitoring to prevent operation in fire or flood. | 676 |  |  |  |  |  |  |  |
|  |  | Isolation of batteries. | 677 |  |  |  |  |  |  |  |
|  |  | Stop of battery charging on loss of ventilation. | 678 |  |  |  |  |  |  |  |
|  |  | Failure modes of control system. | 679 |  |  |  |  |  |  |  |
|  |  | Protection against inadvertent operation. | 680 |  |  |  |  |  |  |  |
| Emergency Shut Downs | ESDs | No single button for ESD (total shutdown). | 681 |  |  |  |  |  |  |  |
|  |  | No direct acting ESD (total shutdown) at external locations. | 682 |  |  |  |  |  |  |  |
|  | F & G | Takes executive action or alarm only. | 683 |  |  |  |  |  |  |  |
|  |  | If takes action, are the means to confirm fire robust e.g. combination of different types of detectors. | 684 |  |  |  |  |  |  |  |
|  |  | Does action match split in redundancy concept. | 685 |  |  |  |  |  |  |  |
|  |  | Does ventilation system introduce a common point in terms of smoke or gas ingestion. | 686 |  |  |  |  |  |  |  |
|  | Thruster Emergency Stops | Hardwired. | 687 |  |  |  |  |  |  |  |
|  |  | Available within easy reach of DPO. | 688 |  |  |  |  |  |  |  |
|  |  | Button layout representative of thruster locations. | 689 |  |  |  |  |  |  |  |
| Emergency Shut Downs | Thruster Emergency Stops | Line monitored. | 690 |  |  |  |  |  |  |  |
|  |  | Alarm on failure of E-Stop power. | 691 |  |  |  |  |  |  |  |
|  | Group Emergency Stops | Split along lines of redundancy concept. | 692 |  |  |  |  |  |  |  |
|  |  | Failure modes of actuators and valves. | 693 |  |  |  |  |  |  |  |
|  |  | Failure modes of control system. | 694 |  |  |  |  |  |  |  |
|  |  | Cable route considerations. | 695 |  |  |  |  |  |  |  |
|  |  | Protection against inadvertent operation. | 696 |  |  |  |  |  |  |  |
|  |  | Cause and effects for fans and flaps. | 697 |  |  |  |  |  |  |  |
|  | Fans | Effect on loss of ventilation. | 698 |  |  |  |  |  |  |  |
|  |  | Ventilation provided following redundancy concept. | 699 |  |  |  |  |  |  |  |
|  |  | Alarms for compartment temperature. | 700 |  |  |  |  |  |  |  |
|  | Flaps | Failure effect on loss of actuation power and control power. | 701 |  |  |  |  |  |  |  |
| Emergency Shut Downs | Flaps | Link to ESD and F&G. | 702 |  |  |  |  |  |  |  |
|  |  | Remote indication. | 703 |  |  |  |  |  |  |  |
|  |  | Protection against inadvertent operation. | 704 |  |  |  |  |  |  |  |
|  |  | Effect of pressure drop on engine crankcase pressure detectors. | 705 |  |  |  |  |  |  |  |
|  | Watertight Doors | Actuation power and control power. | 706 |  |  |  |  |  |  |  |
|  |  | Remote indication. | 707 |  |  |  |  |  |  |  |
|  | Fire Fighting Systems | Control system. | 708 |  |  |  |  |  |  |  |
|  |  | Pipe work and valves. | 709 |  |  |  |  |  |  |  |
|  |  | Division between compartments. | 710 |  |  |  |  |  |  |  |
|  |  | Protection against inadvertent operation. | 711 |  |  |  |  |  |  |  |
|  | Quick Closing Valves | Split along lines of redundancy concept. | 712 |  |  |  |  |  |  |  |
|  |  | Commonality between engine rooms. | 713 |  |  |  |  |  |  |  |
| Emergency Shut Downs | Quick Closing Valves | Failure mode of valves. | 714 |  |  |  |  |  |  |  |
|  |  | Protection of valves. | 715 |  |  |  |  |  |  |  |
|  |  | Protection against inadvertent operation. | 716 |  |  |  |  |  |  |  |
| Effects of Fire and Flooding DP Class 3 | Philosophy and Concept | List of compartments, cable trunks etc. associated with DP related equipment. | 717 |  |  |  |  |  |  |  |
|  |  | Cable routes (power and control) – Analysis refers to dedicated cable route drawings provide from DP Class 3 and confirms correct separation to A60 and WT. | 718 |  |  |  |  |  |  |  |
|  |  | Fire subdivisions between redundant equipment. | 719 |  |  |  |  |  |  |  |
|  |  | WT segregation below damaged waterline. | 720 |  |  |  |  |  |  |  |
| Effects of Fire and Flooding DP Class 3 | Philosophy and Concept | Effects of flooding in any compartment or group of compartments associated with large bore pipework. | 721 |  |  |  |  |  |  |  |
|  |  | Pipe routes. | 722 |  |  |  |  |  |  |  |